Scordelis-Lo Roof Validation in Kratos with IGA element

Objective:

Validate the Scordelis-Lo Roof with Kratos IGA Shell3pElement for linear analysis for both strong and weak BC and perform a convergence study.

Validation is performed against Josef M. Kiendl's Dissertation – Benchmark 6.2.1

- Model Description:

Fig.1: Scordelis-Lo Roof - Taken from Josef M. Kiendl's Dissertation

- Results:

Maximum displacement, ie, Displacement Z at the middle of the roof, is taken for the convergence study. For both strong and weak BC, the convergence was achieved. Tables 1,2, and 3 show convergence behaviour for different polynomial degrees.  Figure 1 shows the convergence plot

| Control Point Refinement | Number of Control Points | Displacement Z |
| --- | --- | --- |
| 5 | 8 | -0.16537 |
| 8 | 11 | -0.25836 |
| 10 | 13 | -0.28012 |
| 20 | 23 | -0.29888 |
| 40 | 43 | -0.30044 |

Table 1: Convergence study with polynomial degree 2

| Control Point Refinement | Number of Control Points | Displacement Z |
| --- | --- | --- |
| 5 | 9 | -0.297 |
| 8 | 12 | -0.30031 |

Table 2: Convergence study with polynomial degree 3

| Control Point Refinement | Number of Control Points | Displacement Z |
| --- | --- | --- |
| 0 | 5 | -0.2593 |
| 2 | 7 | -0.294 |
| 5 | 10 | -0.3007 |

Table 3: Convergence study with polynomial degree 4

Figure 1: Combined Convergence Plot